Solution 5.2.2: Optimization, Search & Exploration 2

Solution

p(x)

x

L

Ay

By

z

dx

z-x

M(z)

Objective function: relation M(z)

With the 2 equilibrium conditions:

(1)

(2)

We find AY and BY:

(1)

(2)

with

Calculation of M(z)  we use the following equilibrium condition:

and M’(z) = dM(z)/dz

- Finding the optimum using the Gradient Method

# Reminder: Gradient Method

xk+1 = xk+f ‘(xk) where  is the coefficient that defines the increment

The load polynomial and the moment diagram for the two cases are demonstrated below:

| Case 1 : Loading 0 0.5 1 1.5 2 2.5 3 3.5 4 4.5 0 2 4 6 8 10 x[m] p(x) [kN/m] Load | Moments 0 1 2 3 4 5 6 7 8 9 10 11 12 13 0 2 4 6 8 10 x [m] Moments [kNm] Moments 1 global maximum |
| --- | --- |
| Case 2: Loading -10 0 10 20 30 40 50 60 0 2 4 6 8 10 x [m] p(x) [kN/m] Load | Moments 0 5 10 15 20 25 30 35 40 45 50 55 60 0 2 4 6 8 10 x [m] Moments [kNm] Moments 2 optima / 1 global maximum |

- Results calculated using a spreadsheet

| Case |  | xo | Convergence? | Global Maximum? | Optimum Value Position x [m] |
| --- | --- | --- | --- | --- | --- |
| 1 | 0.1 | 0 | Yes (but slow) | Yes | 3.71 |
| 1 | 0.5 | 0 | Yes | Yes | 3.76 |
| 1 | 0.5 | 6 | Yes | Yes | 3.76 |
| 2 | 0.1 | 2 | Yes | No | 1.39 |
| 2 | 0.5 | 2 | No (oscillation) | / | / |
| 2 | 0.5 | 0 | No (divergence) | / | / |
| 2 | 0.1 | 6 | Yes | Yes | 7.98 |
| 2 | 0.2 | 6 | Yes | No | 1.39 |

- The weaknesses of this method are:

- choice of  is empirical,

- if  is too small, the number of iterations can be too high,

- if  is too big, there is the risk of oscillating (or diverging) around the optimum without converging,

- in the case of several optima, there is no guarantee that the optimum that is identified is a global optimum.

- There are many other numerical methods. Two of them are:

- Tangent Method (Newton)

- Secant Method

| Tangent Method (Newton) | Secant Method |
| --- | --- |
| x0 x1 f(x0) convergence f(x1) | x0 x2 f(x0) convergence x1 f(x1) |

A weakness of the Tangent Method is that it stops when it meets a point where the value of the derivative of the function equals zero. The Secant Method requires two starting points. Also, it is often slower than the Tangent Method. Note that the problems of oscillation, divergence and non-identification of the optimum remain.

Acknowledgements

Thanks to Y.Robert-Nicoud, S.Saitta, L.Rhode-Barbarigos, S.Korkmaz and E.Hofmans.

I.F.C.Smith, 2011