Exercise 5.5.2: Optimization, Search & Exploration 2

In this exercise set, you will see:

- How to formulate an objective function.

- How to find the optimum of an objective function using the Gradient Method.

Exercise: Optimization without constraints

Assume that a simply supported beam of length L is subjected to an external load of p(x). The load, p(x), is defined by a 2nd-order polynomial.

p(x)

x

L

Direction +

An engineer wants to calculate the position of the maximum moment.

Approach:

- Determine the Objective Function (moment as a function of x) as a function of the polynomial coefficients and the length of the beam.

- Determine the position of the maximum moment using the Gradient Method for the cases given below:

xk+1 = xk+f'(xk)  Formula to find the local maximum

Create a spreadsheet to evaluate xk+1 in increments of 0.25m. Visualize the results using a graph in the spreadsheet. The following table is an example of a spreadsheet template:

| Xi | X[m] | M(Xi) | M’(Xi) |  |
| --- | --- | --- | --- | --- |

The following cases are to be studied:

|  | INPUT |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Case | Given Parameters |  |  |  | Parameters to Vary |  |
|  | a | b | c | L | α | xo |
| 1 | 0.1 | -1.2 | 4.0 | 10 | 0.1 | 0 |
|  |  |  |  |  | 0.5 | 0 |
|  |  |  |  |  | 0.5 | 6 |
| 2 | 1.6 | -13 | 20 | 10 | 0.1 | 2 |
|  |  |  |  |  | 0.5 | 2 |
|  |  |  |  |  | 0.5 | 0 |
|  |  |  |  |  | 0.1 | 6 |
|  |  |  |  |  | 0.2 | 6 |

Questions:

1. For each case, what can be said about:

- Convergence?

- The optimum value that is found (local, global)?

2. What are the weaknesses of the Gradient Method?

- What other numerical gradient method(s) (with which you are already familiar) could be used for to find the maximum moment?

Acknowledgements

Thanks to Y.Robert-Nicoud, S.Saitta, L.Rhode-Barbarigos, S.Korkmaz and E.Hofmans.

I.F.C.Smith, 2011